NCR RANGER HELMET

Electronics Build Document

Version 3.4  |  Galactic Armory Model

# 1. Project Overview

Full electronics integration for a 3D-printed Galactic Armory NCR Ranger helmet. Six integrated systems spanning thermal management, voice processing, ambient hearing, camera/display, and ventilation. Budget $200+.

## 1.1 System Summary

| System | Description | Status |
| --- | --- | --- |
| Fan A | Electronics extraction — right side cavity | Locked |
| Fan C Right | Anti-fog defogging — right cheek cavity, duct to visor | Locked |
| Fan C Left | Crown ventilation — left cheek cavity, silicone tube to crown pads | Locked |
| Voice Chain (A) | Voice amplification + modification — chin speaker | Locked |
| Ambient Hearing (B) | Stereo ambient — wired IEMs via analog preamp chain | Locked |
| Camera + Display | OV2640 camera + GC9A01 round display | Locked |
| Power | 2S2P 18650 battery pack, dual 5V rails | Locked |

## 1.2 MCU

| Parameter | Value |
| --- | --- |
| Board | Arduino Nano ESP32 ABX00083 |
| SoC | ESP32-S3, dual-core LX7 @ 240MHz |
| RAM / Flash | 512KB SRAM / 16MB Flash |
| Connectivity | WiFi, Bluetooth 5.0 |
| Form Factor | 45×18mm Nano footprint |
| Interface | USB-C, hardware I2S, native DVP camera |
| Cost | ~$20 |

# 2. Fan Systems

## 2.1 Fan A — Electronics Extraction

| Parameter | Value |
| --- | --- |
| Fan | WINSINN 2507 — 25×25×7mm, 5V |
| Location | Right side wall electronics cavity |
| Opening | 25.5×19.6mm (confirmed fit) |
| GPIO | GPIO40 — PWM via MOSFET |
| Rocker Switch | NONE — thermal protection always active |
| Sensor | ESP32-S3 internal temperatureRead() |
| CFM | 2.81 CFM (open cavity) |

Fan A Thermal Curve

| Chip Temp | Speed | Hysteresis |
| --- | --- | --- |
| < 50°C | Off | — |
| 50–60°C | 25% | 3°C |
| 60–70°C | 55% | 3°C |
| 70–80°C | 80% | 3°C |
| > 82°C | 100% + LCD warning | 3°C |

## 2.2 Fan C Right — Anti-Fog Defogging

Right cheek 5015 radial blower provides directed anti-fog airflow from the cheek grille upward across the visor interior face. Fan B (Noctua NF-A6x25 crown) was removed — crown position recirculates warm exhaust air and creates hair-ingestion risk.

| Parameter | Value |
| --- | --- |
| Fan | 5015 radial blower, 5V |
| Location | Right cheek cavity (74mm diameter, 25mm deep) |
| Power Rail | 5V Fan rail (Buck 2) |
| GPIO | GPIO41 — shared PWM with Fan C Left |
| Rocker Switch | YES — one rocker kills both Fan C units |
| Airflow Direction | Inlet through right cheek grille → PETG duct channel → upward across visor face |
| Duct cap | Unique to right side — curves toward visor. Same adapter ring body as left. |

## 2.3 Fan C Left — Crown Ventilation

Left cheek 5015 radial blower feeds a silicone tube distribution system running across the helmet crown interior. Air is delivered directly to the scalp through perforations between pad sets — a forced-air motorcycle-style ventilation system.

| Parameter | Value |
| --- | --- |
| Fan | 5015 radial blower, 5V |
| Location | Left cheek cavity (74mm diameter, 25mm deep) |
| Power Rail | 5V Fan rail (Buck 2) — shared with Fan C Right |
| GPIO | GPIO41 — shared PWM with Fan C Right |
| Rocker Switch | YES — one rocker kills both Fan C units |
| Airflow Direction | Inlet through left cheek grille → PETG elbow adapter → silicone crown tube → scalp vents |
| Duct cap | Unique to left side — curves to 8mm tube barb. Same adapter ring body as right. |

## 2.4 Crown Ventilation Tube

| Parameter | Value |
| --- | --- |
| Tube material | Silicone — 8mm ID × 10mm OD |
| Length | ~400–450mm — left cheek cavity to sealed right-side end |
| Path | Left cheek → PETG elbow → across crown interior between pad rows |
| Vent holes | 2–3 × 4mm holes per pad gap — ~6 total vent points across crown |
| Vent angle | Angled slightly downward toward scalp |
| End cap | PETG press-fit plug — sealed end on right side |
| Source | Amazon — uxcell 8mm ID × 10mm OD silicone tubing, 3m roll ~$9 |
| Wall collapse risk | None — 5015 is pushing (pressure), not pulling (suction) |

## 2.5 Fan C Shared Control

| Parameter | Value |
| --- | --- |
| Sensor | BME280 temperature + humidity |
| Startup delay | 10 seconds at power-on before fans spin up |
| Minimum speed | 15% — never goes below this while powered |
| Both fans | Run in sync — complementary functions, same curve appropriate for both |

Fan C Thermal/Humidity Curve

| Condition | Speed | Hysteresis |
| --- | --- | --- |
| 0–10s startup | 0% | — |
| Baseline (power on) | 15% floor — never lower while powered | — |
| Temp > 28°C OR RH > 55% | 35% | 3°C / 5%RH |
| Temp > 32°C OR RH > 65% | 60% | 3°C / 5%RH |
| Temp > 36°C OR RH > 75% | 85% | 3°C / 5%RH |
| Temp > 40°C OR RH > 85% | 100% + LCD warning | 3°C / 5%RH |

Fan C Startup Sequence

| Time | Event |
| --- | --- |
| 0s | Power on — LCD boot screen, all fans off |
| ~2s | BME280 first reading taken, discarded |
| ~4s | Second reading taken |
| ~6s | Third reading taken, averaged with second |
| ~8s | Fan A thermal check begins, status screen populates |
| 10s | Fan C Left + Right spin to 15% minimum — normal operation begins |
| 10s+ | All systems active, curve logic running |

## 2.6 Fan C Adapter Ring — Print Specs

Standard 5015 blowers have a square 50×50mm frame. The cheek cavity is circular 74mm diameter. A two-piece PETG adapter is required per side. The adapter ring body is identical for both sides — only the duct cap differs.

| Part | Spec |
| --- | --- |
| Piece 1 — Adapter ring OD | 73.6mm (0.4mm clearance in cavity) |
| Piece 1 — Inner square cutout | 50.4×50.4mm (0.2mm clearance on 5015 body) |
| Piece 1 — Depth | 20mm (5mm floor clearance) |
| Piece 1 — Material | PETG |
| Piece 1 — Infill | 40% at duct walls, 20% elsewhere |
| Piece 1 — Perimeters | 4 minimum |
| Piece 2R — Right duct cap | Curves outlet upward toward visor face — anti-fog path |
| Piece 2L — Left duct cap | Curves outlet to 8mm round tube barb — crown ventilation path |
| Duct channel length | ~35–45mm up interior shell wall |
| Outlet taper | Wider at base, narrower at outlet to accelerate airflow |
| Wall thickness | 2mm minimum |
| Layer height | 0.15mm for smooth airflow interior |
| Clip interface | 2× M2×8mm screws into adapter ring bosses |
| Shell bond | E6000 along back face, 24hr cure |
| Left elbow adapter | PETG — transitions 5015 rectangular outlet (~12×6mm) to 8mm round tube barb |

# 3. Voice Amplification & Modification (Chain A)

Mono voice output from a chin-mounted speaker. Voice is captured internally, processed by the ESP32-S3 on Core 0, and output through the PAM8610 amp to a single forward-facing speaker.

## 3.1 Components

| Component | Part | Notes |
| --- | --- | --- |
| Microphone | PUI AOM-5024L-HD-R | 14dBA noise floor — DigiKey preferred |
| Codec | PCB Artists ES8388 module | I2S + I2C, ESP-ADF native support. I2C address 0x10 or 0x11 (configurable). Library: arduino-audio-tools ES8388Stream |
| Amplifier | PAM8610 | 2×15W class-D. Cap software volume at 60% to protect speaker (3W max). Fed directly from battery — requires 7V+ minimum |
| Speaker | Visaton K50-8 | Round frame 50mm OD, 17mm depth, 50g, 8Ω, 2W RMS / 3W max, 83dB, IP65, 250–10kHz |
| Speaker location | Chin cup | Faces forward/outward, acoustically isolated from voice mic |

## 3.2 Speaker Cavity

| Parameter | Value |
| --- | --- |
| Cavity OD | 56.9mm maximum (2.24 inches) |
| Cavity depth (original) | 25mm — no modification needed |
| Speaker OD | 50mm — 6.9mm clearance, foam gasket fills gap |
| Speaker depth | 17mm — 8mm clearance in original 25mm cavity |
| Gasket | 1mm closed-cell foam around speaker rim |
| Acoustic isolation | PETG baffle wall separates speaker cavity from voice mic pocket — prevents feedback loop |
| IP65 note | Front-side IP65 rating provides sweat and humidity protection |

## 3.3 Voice Presets

Preset selected via 10K linear pot on GPIO1. Four presets crossfaded by pot position. Core 0 handles all DSP.

| Preset | Effect |
| --- | --- |
| Deep | Pitch −6 semitones |
| Robotic | Pitch shift + 180Hz ring modulation |
| Echo | Pitch shift + 300ms delay |
| Gravel | Pitch shift + soft-clip overdrive |

# 4. Ambient Hearing (Chain B — Fully Analog, Stereo)

Zero-latency stereo ambient hearing via wired IEMs. Fully analog — no MCU involvement. Exterior capsules feed a stereo preamp board which outputs to a 3.5mm panel jack inside the helmet. IEMs plug in before wearing. Left and right channels are completely independent.

## 4.1 Components

| Component | Part | Notes |
| --- | --- | --- |
| Exterior mics | CMA-4544PF-W ×2 | 9.7mm OD, omni, 14dBA noise floor. DigiKey P/N: 102-1721-ND, ~$0.67 each |
| Stereo preamp | NE5532-based stereo mic preamp board | Provides capsule bias voltage, amplifies to headphone level. ~$4–6, 5V operation. Replaces MAX9814 ×2 + Dayton CE38MB-32 ×2 |
| Output jack | 3.5mm panel mount stereo jack | Mounted inside helmet near ear, accessible for IEM plug-in before wearing |
| IEMs | Wired IEMs — dedicated build pair | Over-ear cable routing, foam tips, single dynamic driver. Suggested: KZ ZSN Pro, Moondrop Chu II, or 7Hz Salnotes Zero (~$20–40) |
| Channels | TRUE STEREO | Left capsule to left ear, right capsule to right ear |

## 4.2 Signal Chain

CMA-4544PF-W (×2) → NE5532 stereo preamp board → 3.5mm panel jack → wired IEMs

The preamp board provides the bias voltage the CMA-4544PF-W capsules require (~1–5V through a bias resistor), amplifies the signal to headphone-appropriate level, and drives the IEM load directly. No MAX9814, no ear drivers, no ear cups.

## 4.3 Mic Mounting — Right Side

| Parameter | Value |
| --- | --- |
| Location | Recessed pocket at temple area |
| Vent hole | 5mm through shell wall |
| Orientation | 90° right — faces outward |
| Mount method | Foam gasket in recessed pocket |

## 4.4 Mic Mounting — Left Side

| Parameter | Value |
| --- | --- |
| Location | Small half-circle opening in left cheek grille assembly |
| Opening dimensions | ~9.8×17mm — existing shell feature |
| Capsule fit | 9.7mm OD capsule press-fits directly — no bracket needed |
| Orientation | ~90° left with slight rearward angle |
| Vertical position | Ear canal level |
| Mount method | 1.5mm craft foam gasket around capsule rim, E6000 or silicone seal |

## 4.5 3.5mm Jack Placement

| Parameter | Value |
| --- | --- |
| Position | Interior shell wall near ear — accessible while wearing |
| Cable routing | IEM cable hangs inside helmet, plug-in before putting on |
| Shell penetration | Small grommet or printed boss through shell wall |
| Wire dress | Preamp board wires routed and secured with cable tie anchors |

# 5. Camera & Display

| Component | Part | Notes |
| --- | --- | --- |
| Camera | Arducam OV2640 2MP DVP CCM | ~$8, native DVP to ESP32-S3 LCD-CAM, on-chip JPEG |
| Display | GC9A01 1.28" round IPS 240×240 | ~$6, SPI, TFT_eSPI library |
| Toggle switch | GPIO39 INPUT_PULLUP | Camera off = status screen; Camera on = live feed |
| Camera MOSFET | GPIO38 | Gates camera power + OV2640 RESET. 0mA draw when off |
| Frame rate | ~10fps at 320×240 JPEG | Core 1 handles capture |

## 5.1 Status Screen (Camera Off)

Displays: Chip °C, Ambient °C/RH, Fan A %, Fan C %, Voice preset, Battery %.

# 6. Complete GPIO Assignment

| GPIO | Function | Notes |
| --- | --- | --- |
| GPIO1 (A0) | Voice preset pot | 10K linear |
| GPIO2 | Status LED PWM |  |
| GPIO4 | I2S BCLK → ES8388 |  |
| GPIO5 | I2S WCLK → ES8388 |  |
| GPIO6 | I2S DACDAT → ES8388 |  |
| GPIO7 | I2S ADCDAT ← ES8388 |  |
| GPIO8 | I2C SDA | ES8388 (0x10/0x11), BME280 (0x76), OV2640 — shared bus |
| GPIO9 | I2C SCL | Shared bus |
| GPIO10 | OV2640 XCLK | DVP camera clock |
| GPIO11 | OV2640 PCLK |  |
| GPIO12 | OV2640 VSYNC |  |
| GPIO13 | OV2640 HREF |  |
| GPIO14–21 | OV2640 D0–D7 | DVP data bus |
| GPIO33 | GC9A01 CS | SPI display |
| GPIO34 | GC9A01 DC |  |
| GPIO35 | GC9A01 RST |  |
| GPIO36 | GC9A01 MOSI |  |
| GPIO37 | GC9A01 SCK |  |
| GPIO38 | Camera MOSFET + OV2640 RESET | Gates camera rail |
| GPIO39 | Camera toggle | INPUT_PULLUP |
| GPIO40 | Fan A PWM | WINSINN 2507 electronics extraction |
| GPIO41 | Fan C PWM | Both 5015 cheek blowers in parallel (Fan C Right + Fan C Left) |
| GPIO42 (A2) | RESERVED | Spare — formerly Fan B pot |

# 7. Power Architecture

| Rail | Source | Devices | Est. Peak Draw |
| --- | --- | --- | --- |
| 5V Main | Buck 1 (original) | ESP32-S3, ES8388, NE5532 preamp, BME280, GC9A01, OV2640, Fan A | ~770mA |
| 5V Fan | Buck 2 (retuned from 7V) | Fan C Right 5015, Fan C Left 5015 | ~400–800mA |
| Battery direct (7.4V nom) | 2S2P pack — no buck | PAM8610 amp only | ~500mA at moderate volume |

| Parameter | Value |
| --- | --- |
| Battery | 4× 18650 in 2S2P configuration |
| Nominal voltage | 7.4V (8.4V fully charged, ~7.0V depleted) |
| Buck 1 | Original 5V buck — electronics rail |
| Buck 2 | Retuned from 7V to 5V — dedicated fan rail |
| PAM8610 supply | Direct from battery (7.4–8.4V) — no buck needed. Rocker switch on battery line to amp |
| Rail isolation | Fan PWM noise isolated from MCU and audio. Amp switching noise on its own direct battery line |
| Battery placement | 220g battery shifted 20–30mm LEFT of centerline — offsets right-side electronics weight bias. Consider rearward shift to counterbalance chin speaker weight |
| MOSFETs | IRLML6244 or 2N7002 ×4 — same part throughout (Fan A, Fan C, Camera, spare) |

## 7.1 Rocker Switch Assignment

| Switch | Controls | Notes |
| --- | --- | --- |
| Voice (codec) | ES8388 codec + speaker mic — 5V Main rail |  |
| Voice (amp) | PAM8610 amp — battery direct rail | Separate rocker — amp needs 7V+ minimum |
| Ambient | NE5532 preamp board + 3.5mm jack | Wired IEMs — fully analog chain |
| Fan C | Both 5015 cheek blowers (Right + Left) | One switch kills both |
| Fan A | NO ROCKER | Thermal protection always active |
| Camera + Display | Software toggle via GPIO39 | No rocker — draws from always-on 5V Main |

# 8. Thermal & Humidity Sensing

| Parameter | Value |
| --- | --- |
| Primary sensor | BME280 breakout — I2C address 0x76 |
| Measures | Temperature, humidity, pressure |
| Location | Lower rear interior, 40mm+ from Fan A exhaust port |
| Used by | Fan C curve (temp + humidity), LCD status screen |
| Chip temp sensor | ESP32-S3 internal temperatureRead() — Fan A curve only |
| Startup | First reading discarded, second + third averaged for initial condition assessment |

# 9. Shell Print Specifications

| Zone | Wall Thickness | Perimeters | Infill | Material |
| --- | --- | --- | --- | --- |
| General shell | 4–5mm | 4 | 15–20% | PLA |
| Electronics wall (right) | 5–6mm | 5 | 40% | PLA |
| Fan A port surround | 6–8mm | 6 | 60% | PLA |
| Anchor boss zones | 8–10mm | 6+ | 80%+ | PETG inserts mandatory |
| Crown / ventilation zone | 5–6mm | 5 | 40% | PLA — crown tube routes along interior |
| Chin / speaker cavity | 4–5mm | 4 | 20% | PLA — no depth modification needed for K50-8 |
| Camera port | 3–4mm | 4 | 30% | PLA |
| Cheek 5015 adapter rings | N/A — separate | 4 | 20–40% | PETG |

## 9.1 PETG Anchor Inserts

| Parameter | Value |
| --- | --- |
| Quantity | 4 inserts |
| Perimeters | 8 |
| Infill | 100% |
| Material | PETG mandatory |
| Pocket clearance | 0.2mm |
| Flange lip | 4–5mm |
| D-rings | 20mm stainless steel |
| Hardware | M3×16mm SS bolts + nyloc nuts |
| Backing plates | 3mm aluminium (right side only — electronics weight bias) |
| Adhesive | JB Weld epoxy, 24hr cure before any load |

# 10. MICH/ACH Padding & Y-Strap

| Parameter | Value |
| --- | --- |
| Pad kit | Oregon Aero / Team Wendy — 7-pad, all Velcro-backed |
| Y-strap | 2 anchors per side → 25mm tri-glide → ITW Nexus SR buckle → XRD chin cup |
| Right side pad | Sized to clear electronics stack and Fan A exhaust port |
| Crown pads | Silicone ventilation tube routes between pad sets — 2–3 vent holes per pad gap |
| Total weight (excl. shell) | ~605g (electronics ~425g, pads + strap ~180g) |

# 11. Bill of Materials

| Item | Part / Source | Qty | Est. Cost |
| --- | --- | --- | --- |
| MCU | Arduino Nano ESP32 ABX00083 | 1 | ~$20 |
| Voice mic | PUI AOM-5024L-HD-R (DigiKey) | 1 | ~$4 |
| Codec | PCB Artists ES8388 (pcbartists.com / Tindie) | 1 | ~$17 |
| Amplifier | PAM8610 module | 1 | ~$3 |
| Speaker | Visaton K50-8 (Parts Express / Amazon) | 1 | ~$8 |
| Ambient mics | CMA-4544PF-W (DigiKey 102-1721-ND) | 2 | ~$1.50 |
| Stereo preamp board | NE5532-based stereo mic preamp (Amazon / AliExpress) | 1 | ~$5 |
| 3.5mm panel jack | 3.5mm panel mount stereo jack (Amazon / DigiKey) | 1 | ~$1 |
| Wired IEMs | KZ ZSN Pro / Moondrop Chu II / 7Hz Salnotes Zero | 1 | ~$25 |
| Camera | Arducam OV2640 2MP DVP (Amazon / Arducam) | 1 | ~$8 |
| Display | GC9A01 1.28" round IPS (Amazon / AliExpress) | 1 | ~$6 |
| Fan A | WINSINN 2507 5V (Amazon) | 1 | ~$5 |
| Fan C Right | 5015 radial blower 5V — anti-fog (Amazon / AliExpress) | 1 | ~$5 |
| Fan C Left | 5015 radial blower 5V — crown ventilation (Amazon / AliExpress) | 1 | ~$5 |
| Silicone crown tube | uxcell 8mm ID × 10mm OD silicone tube 3m roll (Amazon) | 1 | ~$9 |
| PETG elbow adapter | Print — 5015 outlet to 8mm tube barb (own filament) | 1 | ~$1 |
| PETG end cap | Print — crown tube sealed end (own filament) | 1 | ~$1 |
| BME280 | BME280 I2C breakout (Amazon / Adafruit) | 1 | ~$4 |
| Battery cells | 18650 Li-ion ×4 | 4 | ~$20 |
| BMS | 2S BMS board (Amazon / AliExpress) | 1 | ~$5 |
| Buck converters | 5V buck ×2, 2A+ rated (Amazon / AliExpress) | 2 | ~$6 |
| MOSFETs | IRLML6244 or 2N7002 ×4 (DigiKey / Amazon) | 4 | ~$2 |
| Rocker switches | Mini rocker ×4 (Amazon) | 4 | ~$4 |
| Mesh | Stainless woven wire 30–40 LPI 12×12" (Amazon / McMaster 9225T61) | 1 | ~$11 |
| Pads + strap | Oregon Aero / Team Wendy 7-pad kit | 1 | ~$40 |
| PETG filament | Adapter rings + duct channels + inserts + elbow + end cap | — | ~$5 |
| Hardware | M3 bolts, nyloc nuts, D-rings, backing plates | — | ~$8 |
| Velcro | 25mm Velcro strip — hook + loop (Amazon) | 1 | ~$3 |
| Adhesives | E6000, JB Weld, foam gasket material | — | ~$10 |
|  | ESTIMATED TOTAL |  | ~$242 |

# 12. Pending Items

- ESP32-S3 voice sketch rewrite — hardware I2S at 44.1kHz, ES8388 arduino-audio-tools ES8388Stream library, dual-core architecture (Core 0: DSP, Core 1: camera capture)

- 5015 blower — confirm specific part number with round inlet geometry before ordering

- Acoustic baffle wall design — PETG divider between speaker cavity and voice mic pocket

- Fan C Right duct cap — final geometry, confirm cheek-to-visor distance on physical print

- Fan C Left duct cap + elbow adapter — curve to 8mm tube barb, confirm fit before left adapter ring print

- Crown tube vent hole placement — confirm pad gap positions on physical shell before drilling

- NE5532 preamp board selection — confirm specific board, verify bias voltage output suits CMA-4544PF-W, test signal level into IEMs before mounting

- IEM selection — wired, over-ear cable, foam tips, single dynamic driver, ~$20–40

- 3.5mm jack placement — confirm accessible position inside shell near ear canal level

# 13. Version History

| Version | Changes |
| --- | --- |
| v1.0 | Initial build — basic fan, voice, ambient hearing, power spec |
| v2.0 | Added ESP32-S3 MCU, OV2640 camera, GC9A01 display, dual-core architecture |
| v2.1 | Added dual-fan thermal management, BME280, Fan B crown ventilation |
| v3.0 | Fan B (crown) removed. Fan C cheek 5015 anti-fog system added. Speaker upgraded to Visaton K50-8 + PAM8610. Left mic mount finalised. Dual 5V rail power architecture. Fan C startup delay and 15% minimum speed floor added. |
| v3.1 | PAM8610 corrected to direct battery feed (7V+ minimum). 5th rocker added for amp rail. |
| v3.2 | ES8388 replaces WM8960. K50-8 replaces FRS5-8. I2C addresses updated. |
| v3.3 | Added Section 6 — Ear Cups. PETG circumaural cups, Velcro-mounted, CE38MB-32 driver, memory foam gasket. Left cup offset 10mm for 5015 clearance. |
| v3.4 | Fan C split: Right cheek = anti-fog defogging (unchanged). Left cheek = crown ventilation via silicone tube + pad gap vents. Ear cups, Dayton CE38MB-32 drivers, MAX9814 boards removed. Ambient hearing simplified: CMA-4544PF-W capsules → NE5532 stereo preamp → 3.5mm jack → wired IEMs. Rocker count reduced from 5 to 4. Camera + Display moved to software-only toggle. Sections renumbered. BOM updated ~$242. |

Document ends — v3.4